When handling knives, box cutters, or machines and tools with blades, take care to protect yourself from cuts. Cuts from knives may occur as a result of improper handling, improper storage, or the accidental dropping of knives. To prevent injury, apply the following approaches.

Planning:

- Assess tasks to identify the potential hazards and appropriate personal protective equipment (PPE).

- Require employees to use appropriate controls and hand protection when exposed to hazards such as cuts and lacerations.

- Review any hazards and needed controls for wearing gloves around machinery if there is a risk that gloves may be pulled into the machinery.

- Equip newly purchased knives and cleavers with blade guards or knuckle guards that protect the hand from slipping onto the blade.

- Allow only experienced, trained and authorized employees to handle and sharpen blades.

Proper usage:

- Wear cut-resistant gloves to minimize cut hazards when handling sharp items.

- Cut properly:

  - Cut away from the body.

  - Keep fingers and knuckles away from the blade. Assure that hands are out of the path of travel.

- Carry knives properly:

- With the tip pointed down to your side.

- With the cutting edge angled slightly away from the body.

- Do not try to catch a falling knife.

- Stay focused while cutting. Do not talk with other employees while using a knife or sharp item. When interrupted, stop working.

Proper usage (continued):

- Use a knife only for its intended purpose, and use the appropriate knife for the cutting job.

- When handing a knife or sharp item to someone, place it down on a clean surface, and let the other person pick it up.

Storage:

- Do not store knives or sharp items where they could be accidently bumped, such as near the edges of work tables.

- Store knives, saws, and sharp items in a designated storage area when not in use. Do not store the blades with the cutting edge exposed.

Blade care:

- Keep knives and blades sharp. Dull blades require more force, and they are more likely to slip under higher force, presenting a risk of laceration.

- Immediately clean knives after each use or place them in a dishwasher or a dishpan labeled “for knives only.”

- Do not store knives and sharp objects in a sink between periods of use.

This form documents that the training specified above was presented to the listed participants. By signing below, each participant acknowledges receiving this training.

Organization:							Date:

Trainer:				          	 Trainer’s Signature:

Class Participants:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature: