Combustible dust is defined as a solid material composed of distinct particles which present a fire hazard when suspended in air. A dust explosion can cause catastrophic loss of life, injuries, and destruction of property.

Characteristics of a dust explosion:

- Definition: A dust explosion can be defined as a rapid release of high pressure gas into the environment.

- Process:

- It commonly begins with the ignition of a fuel, such as a combustible dust, that burns very rapidly.

- It produces a large and sudden release of gas and does not necessarily involve a fire.

- There are often more serious secondary explosions that result from an initiating explosion
which dislodges loose particulates.

- Five criteria that must be present for dust explosion conditions:

- Fuel, i.e., combustible dust

- Heat or an ignition source

- Oxygen, which may be in the form of an oxidizer chemical

- Dispersion of dust particles of a certain concentration and quantity

- A confined dust cloud

Examples of industries where combustible dusts are common:

- Grain handling

- Flour production

- Industries using wood

- Sugar manufacturing

- Certain types of plastic

- Coal handling

- Textile industries using cotton, nylon, etc.

- Metal production such as aluminum, sodium, and potassium

Fighting dust accumulation:

Assure that dust-containing systems, ducts, and dust collectors are designed in a manner that prevents fugitive dusts from accumulating in the work area. This includes proper air velocity and enough volume in the dust collectors for the various applications.

  - Assure that the working surfaces are designed in a manner to minimize dust accumulation and to facilitate cleaning.

  - Accumulations on overhead beams, joists, ducts, the tops of equipment, and other surfaces must be included when assessing dust concentrations.

  - Your facility must have a housekeeping program with regular cleaning frequencies established for floors and horizontal surfaces, including ducts, pipes, hoods, ledges, and beams, to minimize dust accumulation within operating areas of the facility.

  - Housekeeping practices must include immediate cleaning whenever a dust layer of 1/32-inch thickness accumulates over a surface area of 5% or more of the floor area of the facility or any given room.

- If the floor area exceeds 20,000 ft2, a dust layer must not exceed a 1,000 ft2 floor area.

Controlling heat and ignition sources:

- Electrical equipment must meet Class II requirements for explosion proof control.  This includes electrically-powered cleaning devices such as vacuum cleaners.

- An effective ignition control program must be in place.

- Include in the program such elements as grounding and bonding, metal detection, preventive maintenance programs, and other methods.

- An effective program will dissipate any sparks or electrostatic charge that could be generated where a combustible atmosphere may exist (e.g., in ductwork).

- Implement a hot work permit program.

- Post and enforce no smoking areas.

- Assure that duct systems, dust collectors, and dust-producing machinery are bonded and grounded to minimize accumulation of static electrical charge and equipped with spark detection and arrest systems.

- Use industrial trucks, machinery, and electrical equipment that are approved for Class II combustible dust locations.

Follow these control measures to avoid dust build-up and prevent dust explosions.

This form documents that the training specified above was presented to the listed participants. By signing below, each participant acknowledges receiving this training.

Organization:							Date:

Trainer:				          	 Trainer’s Signature:

Class Participants:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature:

Name: 				                        Signature: